# Chapter III
DYNAMIC MODEL

This chapter is written to contribute with an outline about the mathematical model of humanoids in which equations are related in matrices. This mathematical model relates the motion of all joints of a humanoid robot irrespective of the high number of degrees of freedom it might have which are attached to actuator that demand control. These parameters include the position and the velocity of each joint which are results of the dynamic model. In this thesis, the aim is to find a mathematical relation between the position of each joint and the force at each joint which the dynamic model can afford. The force at the joints when calculated is an essential input the controller and will enhance the stability of the robot as it is given the ability to localize and evaluate force with the help of artificial intelligence.

## Kinematics

Kinematics, in common, is the study of the position, speed and increasing speed of a mechanical system without considering either the forces (or torques) that create the movement or the dynamic properties, such as mass or inertia, of the bodies. The joint space of a robot with n DoF is the n-dimensional manifold Q containing all the possible values that the joint variables q can take. For robotic manipulators, the position end-effector is the main concern. However, for more generic robots such as humanoids, this can represent any part of the robot whose pose is of interest. The velocity of an operational point x can be represented by spatial velocity ξ that contains both the linear and angular velocities; alternatively, it can be given by the variation of its representation x.

Forward kinematics consists in finding the position x ∈ SE(3) of an operational point given a certain joint angle configuration q ∈ Q, and is described by the map f : Q → SE(3) such that x = f(q). Inverse kinematics aims at finding the joint configuration q ∈ Q that achieves a given pose x ∈ SE(3) for a certain operational point. In some specific cases, this can be represented by the inverse map f−1: SE(3) → Q such that q = f−1(x), but in general f−1(x) is not unique and can even be undefined.

The control objective is defined by a reference point or a reference trajectory in space, and the control algorithm has to determine the appropriate angular position of the joint configuration q(t) that achieves the objective. Consider the task i with mi DoF. An n-DoF robot is said to be kinematically redundant with respect to task i if n > mi, and n − mi is known as the degree of redundancy with respect to task i.

## Dynamics

Dynamics is the study of the relativity between the robot motion and the generalized forces that act on the robot generating that motion. This relation considers factors such as lengths, masses and inertia of the elements composing the robot.  In the dynamic model, the motion is signified through joint variables acceleration q, or operational points acceleration x. If the joints are revolute joints, the generalized forces are equivalent to the joint torques, and if the joints are prismatic joints they are the joint forces.

There are two main problems in dynamics:  Forward Dynamics expresses the motion of the robot as a function of the generalized forces applied to it. Whereas, Inverse Dynamics expresses the generalized forces acting on a robot as a function of the robot motion. Two main approaches to computing a humanoid’s dynamic model; the Newton-Euler method and the Lagrange-Euler Method.

### Newton-Euler

The equations of Newton are essential when it comes to solving a dynamic model of any robot or mechanical system. The equation relates the acceleration under the effect of the summation of forces which is Newton’s second law of motion. Newton's second law of motion for a particle with mass dm exhibiting a velocity x under influence of a net force dF is given by:

| dF=ddt(dmx) | (5) |
| --- | --- |

Since the constraint forces on the robot can be omitted the general equation of motion can be derived by summing the net applied forces of equation 5 over all particles in a body A and assuming a constant mass with respect to time.

| dF=dmx | (6) |
| --- | --- |
| F=mx | (7) |

Moreover, Newton claimed that if body A is exerting a force on body B that particle B at the same time is exerting the opposite force with the same magnitude on particle A, also known as Newton's third law of motion. A rigid body consists of different particles constrained to each other by constraint forces. The common condition of movement for an inflexible body can be determined utilizing Newton's second and third laws of motion and the so called d'Alembert principle. This guideline states that the sum of constraint forces between the particles in a body can be neglected since these forces don't contribute to the acceleration of the body. The constraint forces between the particles don't perform work inside a body. The d'Alembert principle for a body containing N particles i, performing a virtual displacement δx, is given as:

| i=1NdFi- ddtdmixiδx=0 | (8) |
| --- | --- |

In the same way the rotational dynamics of a body are defined in the principle of Euler. This principle describes the angular acceleration φ of a particle with inertia I under the influence of a net moment M:

| M=Iφ | (9) |
| --- | --- |

Lagrange-Euler

This method uses the minimal of set of coordinates needed to describe the system and full the constraints, the so called generalized coordinates. The equations of motion for series of bodies in a Lagrangian way are based on energy. Every body in the series contains energy, generally consisting of kinetic and potential energies. The kinetic energy of a body with mass m and inertia I is described in equation 10 where x and φ are linear and angular velocities respectively.

| K=12mx2+12Iφ2 | (10) |
| --- | --- |

The potential energy consists of different parts, for example gravitational energy or elastic energy. The gravitational energy of the body at height h in a gravitational acceleration g is shown in equation 11. The equation of the elastic potential energy stored in a spring with linear stiffness k and elongation y is shown in equation 12.

| P=mgh | (11) |
| --- | --- |
| P= 12ky2 | (12) |

Using this notation and the d'Alembert principle, the equations of motion of one or multiple bodies can be derived. The Lagrange-Euler equation is given in equation 13 where Qnc is the non-conservative forces such as moments damping and friction.

| ddt∂K∂q- ∂K∂q+∂P∂q=Qnc | (13) |
| --- | --- |

### Application on Humanoid Robots

The robot body consists of sequential links. The base link which is also the first link, denotes the reference frame. The robot is considered as a biped model and the upper body is omitted although it has a great effect on the robot, but only for the ease of the calculations. It is possible to generalize these relations throughout the whole robot body to take into account the effect of upper body. One of the robot feet is considered as the reference base which is to be taken as the starting point or the first frame and then continuing to its shank, thigh and hip links, and afterwards moving inversely in the other leg to reach its foot. The parametric kinematical condition of robot is required for dynamical analysis. The Denavit-Hartenberg method is used, and the DH parameters needed to calculated the Transformation Matrices T for the kinematics serve the dynamic model and analysis. Frame affixing also known as the DH parameters describe the relative position of the frames. Four DH parameters are defined for each intermediate link: joint angle variable q and three constants a, d, α. The frames attached to the joints of robot are shown in figure# and table# contains the DH parameters used in the proposed method with the parameters of each joint. The transform matrix ii-1T which defines the transformation of frame i relative to frame i -1 is:

| ii-1T= r11r12r13r21r22r23r31r32r33abc 0 0 01 | (14) |
| --- | --- |
| ii-1T=cosθi-sinθi0sinθicosαi-1cosθicosαi-1-sinαi-1sinθisinαi-1cosθisinαi-1cosαi-1αi-1bc 0 0 01 | (15) |

This consists of a 3 x 3 rotation matrix and a 3 x 1 position matrix. The transformation in each joint affects the next link and the transformation propagates forward. Another frame will be affixed to the end of the last link in order to determine the location of the other foot. Having spatial transformations of the robot links relative to each other, robot dynamics would be acquired.

Figure 1. Joint Frames for Humanoid Robot

Table 1. DH Parameters

| i | αi-1 | ai-1 | di | θi |
| --- | --- | --- | --- | --- |
| 1 | Malleolus | 0 | 0 | θ1 |
| 2 | π2 | 0 | 0 | θ2 |
| 3 | 0 | Shank | 0 | θ3 |
| 4 | π2 | 0 | Thigh | θ4 |
| 5 | π2 | 0 | 0 | θ5 |
| 6 | π2 | 0 | 0 | θ6 |
| 7 | 0 | 0 | Hip | θ7 |
| 8 | π2 | 0 | 0 | θ8 |
| 9 | -π2 | 0 | Thigh | θ9 |
| 10 | π2 | 0 | 0 | θ10 |
| 11 | 0 | Shank | 0 | θ11 |
| 12 | π2 | 0 | 0 | θ12 |
| 13 | Malleolus | 0 | 0 | 0 |

The Newton-Euler iterative method is to be used since it is computationally better for programing and it consists of two outward (from the base to the end-effector) and inward (form the end-effector to the base) series of iterations

with the following relations, is used.

During outward iterations in computing velocities and accelerations, kinematical parameters of the base are assumed known. Also in inward iterations while computing force and torque of the joints, using the contact model, the contact forces of the non-reference foot, should be defined. In this thesis, the humanoid robot will be experimentally under study when it is affected by external forces and disturbances which will affect the equations. Knowing that the forces are going to be applied at the center of gravity of specified joints, some parameters will have to be added to the equations to take the effect of the external disturbances into consideration.

Outward iterations: i from 0 to n -1 (n is the number of joints)

i+1ωi+1=ii+1R  iωi+ θi+1 i+1Zi+1

i+1ωi+1=ii+1R  iωi+ii+1R  iωi×θi+1 i+1Zi+1+θi+1 i+1Zi+1

i+1vi+1=ii+1R  iωi× iPi+1 + iωi× iωi× iPi+1 + ivi+2 i+1ωi+1×di+1 i+1Zi+1+di+1 i+1Zi+1

i+1vCi+1= i+1ωi+1× i+1PCi+1 + i+1ωi+1× i+1ωi+1× i+1PCi+1 + i+1vi+1

i+1Fi+1=mi+1 i+1vCi+1

i+1Ni+1= Ci+1Ii+1  i+1ωi+1+ i+1ωi+1× Ci+1Ii+1  i+1ωi+1

with,    i+1Zi+1=001T  and θi+1=θi+1= 0  for prismatic joint and di+1= di+1 =0  for revolute joint

0ω0= 0ω0=0 and  0v0=00gT is the fictitious upward acceleration to include the effect of gravity.

Inward iterations: i from n to 1(n is the number of joints)

ifi=i+1iR  i+1fi+1+  iFi

ini= iNi+i+1iR  i+1ni+1+ iPCi× iFi+ iPi+1×i+1iR  i+1fi+1

τi= iniT iZi for revolute joint and   τi= ifiT iZi for prismatic joint

Where,  n+1fn+1= n+1nn+1=0 when there is no external loads on the end-effector.

n	Number of degrees of freedom

q 	Joint variables

x 	Position of an operational point

ξ 	Spatial velocity

δx 	Virtual Displacement

x	Velocity of an operational point

q  	Joint variables acceleration

x  	Acceleration of an operational point

dF 	Net force

φ 	Angular acceleration

I 	Inertia

M 	Net moment

h 	Height

g 	Gravitational Acceleration

Qnc  	Non-conservative forces

jωi 	Absolute angular velocity of the link i relative to the frame of link j

ii+1R 	Rotational matrix from frame i to i+1

θi+1 	Angular velocity of joint i+1

jωi 	Absolute angular acceleration of the link i relative to the frame of link j

θi+1 	Angular acceleration of joint i+1

jvi	Absolute linear acceleration of the link i relative to the frame of link j

jPi 	Position of origin of the link i relative to the frame of link j

ivCi	Absolute linear acceleration of center of mass of link i relative to its own frame

iPCi  	Position of center of mass link i relative to its own frame

iFi 	Inertial force exerted on link i

mi 	Mass of link i

iNi 	Inertial torque exerted on link i

CiIi 	Inertial tensor of link i

ifi 	Force exerted on link i by link i-1

ini 	Torque exerted on link i by link i-1

τi 	Joint torque of link i